## Binary to BCD Converter

### Shift and Add-3 Algorithm

- Shift the binary number left one bit.

- If 8 shifts have taken place, the BCD number is in the Hundreds, Tens, and Units column.

- If the binary value in any of the BCD columns is 5 or greater, add 3 to that value in that BCD column.

- Go to 1.

| Operation | Hundreds | Tens | Units | Binary |  |
| --- | --- | --- | --- | --- | --- |
| HEX |  |  |  | F | F |
| Start |  |  |  | 1 1 1 1 | 1 1 1 1 |

### Example 1: Convert hex E to BCD

### Example 2: Convert hex FF to BCD

### Truth table for Add-3 Module

Here is a Verilog module for this truth table.

module add3(in,out);

input [3:0] in;

output [3:0] out;

reg [3:0] out;

always @ (in)

case (in)

4'b0000: out <= 4'b0000;

4'b0001: out <= 4'b0001;

4'b0010: out <= 4'b0010;

4'b0011: out <= 4'b0011;

4'b0100: out <= 4'b0100;

4'b0101: out <= 4'b1000;

4'b0110: out <= 4'b1001;

4'b0111: out <= 4'b1010;

4'b1000: out <= 4'b1011;

4'b1001: out <= 4'b1100;

default: out <= 4'b0000;

endcase

endmodule

### Binary-to-BCD Converter Module

Here is a structural Verilog module corresponding to the logic diagram.

module binary_to_BCD(A,ONES,TENS,HUNDREDS);

input [7:0] A;

output [3:0] ONES, TENS;

output [1:0] HUNDREDS;

wire [3:0] c1,c2,c3,c4,c5,c6,c7;

wire [3:0] d1,d2,d3,d4,d5,d6,d7;

assign d1 = {1'b0,A[7:5]};

assign d2 = {c1[2:0],A[4]};

assign d3 = {c2[2:0],A[3]};

assign d4 = {c3[2:0],A[2]};

assign d5 = {c4[2:0],A[1]};

assign d6 = {1'b0,c1[3],c2[3],c3[3]};

assign d7 = {c6[2:0],c4[3]};

add3 m1(d1,c1);

add3 m2(d2,c2);

add3 m3(d3,c3);

add3 m4(d4,c4);

add3 m5(d5,c5);

add3 m6(d6,c6);

add3 m7(d7,c7);

assign ONES = {c5[2:0],A[0]};

assign TENS = {c7[2:0],c5[3]};

assign HUNDREDS = {c6[3],c7[3]};

endmodule

### General Binary-to-BCD Converter

The linked code is a general binary-to-BCD Verilog module, but I have not personally tested the code.

Reference: course materials from Prof. Richard E. Haskell

Maintained by John Loomis, last updated 4 Jan 2004