# A mikrokontroller előzetes programja

#ifdef DEVEL

#pragma NOINTVECTOR

#endif

/* use OPTIMIZE 1 or 2 only! */

#pragma OPTIMIZE (3)

#pragma SMALL            /* use internal RAM only */

#pragma LC

#include <type_def.h>    /* definitions of commonly used data types */

#include <reg_port.h>    /* def. of 8052 registers */

#include <blood_io.h>    /* def. of presure meter port bits */

#include <arit.h>        /* BIN-BCD conversion */

#include <sio.h>         /* sio handler */

#include <timers.h>      /* handlers of timers */

#include <thermo.h>      /* temerature sensor handler */

#ifdef DEVEL

#include <mon_it.h>

#endif

/* remote commands: */

/* test of serial line */

#define TEST     'T'

/* motor on: */

#define MOT_ON   'B'

/* motor off: */

#define MOT_OFF  'K'

/* analog szelep szab.:A,data (0-64) */

#define ANAL     'A'

/* szelep0 on: */

#define SZ0_ON   'Z'

/* szelep0 off: */

#define SZ0_OFF  'N'

/* meres inditas, 16 bites eredmeny automatikusan jon, Lo, Hi : */

#define MERES    'M'

/* homerselet meres, 9 byte adat automatikusan jon */

#define HOMERES  'H'

/* folyamatos meres inditas, */

/* 16 bites eredmeny automatikusan jon, Lo, Hi : */

/* leallitas parancsig */

#define CMER_ON  'C'

/* folyamatos meres leallitas */

#define CMER_OFF 'F'

#define ERROR    'E'

/* 280Hgmm-nel a motor mindenkepp leall  */

#define PMAX 0x19A0

signed char tim;

bit  tim_out;

bit  start;

bit  stop;

byte pressl,pressl0;

byte pressh,pressh0;

int press;

idata byte TEMPL;

idata byte TEMPH;

idata byte user1;

idata byte user2;

idata byte res1;

idata byte res2;

idata byte remain;

idata byte per_c;

idata byte crc;

bit  ANAL_EN,CMER;

int  SZ1;

signed char SZ2;

char  KOMP;

#ifdef DEVEL

void TIM2_IT();

void SIO_IT();

#endif

void serou_hom()

{

while(~TR_END){}

serou_leng=9;

SER_OU[0]=TEMPL;

SER_OU[1]=TEMPH;

SER_OU[2]=user1;

SER_OU[3]=user2;

SER_OU[4]=res1;

SER_OU[5]=res2;

SER_OU[6]=remain;

SER_OU[7]=per_c;

SER_OU[8]=crc;

trp=0;

TR_END=0;

TI=1;

}

void serou_press()

{

while(~TR_END){}

serou_leng=2;

SER_OU[0]=pressl0;

SER_OU[1]=pressh0;

trp=0;

TR_END=0;

TI=1;

}

void INIC()

{

char TL;

char TH;

#ifdef DEVEL

MON_IT(5,TIM2_IT);

MON_IT(4,SIO_IT);

#endif

TL=0;

TH=0xFF;

TIM2_INI(TL,TH);

TIM0_INI(0,0);

SIO_INI();

EA=1;               /* enable IT */

start=0;

stop=0;

MOTOR=0;

SZELEP0=0;

SZELEP1=0;

KOMP=0;

ANAL_EN=0;

CMER=0;

}

void TIM2_IT() interrupt 5 using 1

{

TF2=0;      //clear IT flag

if (start)

{

TL0=0;

TH0=0;

TR0=1;

start=0;

stop=0;

SZ1=124;

}

else

{

if (~stop)

{

SZ1-=1;

if (SZ1==0)

{

TR0=0;

pressl=TL0;

pressh=TH0;

SZ1=124;

stop=1;

}

}

}

if (tim>0)

tim--;

else

{

tim_out=1;

}

SZ2=(SZ2+1) & 0x3f;   /* modulo 64 */

if (ANAL_EN)

{

if (SZ2<KOMP)

SZELEP1=0;

else

SZELEP1=1;

}

}

void SIO_IT() interrupt 4 using 2

{

if (TR_END)

{

TI=0;

}

if (RI)

{

sin_wr();

}

else

{

if (TI)

{

transmit();

TI=0;

}

}

}

main() using 0

{

INIC();

while (1)

{

start=1;

while(~stop){};

stop=0;

EA=0;

pressh0=pressh;

pressl0=pressl;

EA=1;

press=pressh0;

press=(int)(press << 8) | pressl0;

if (MOTOR)

{

if (press<PMAX)

{

MOTOR=0;

}

}

S_DAT=0x00;

if (GET_SIO())

{

switch (S_DAT)

{

case TEST:    sout_byte('T');

break;

case MOT_ON:  MOTOR=1;

break;

case MOT_OFF: MOTOR=0;

break;

case SZ0_ON:  ANAL_EN=0;

SZELEP0=1;

break;

case SZ0_OFF: ANAL_EN=1;

SZELEP0=0;

break;

case ANAL:    while (~GET_SIO()){};

KOMP=S_DAT;

ANAL_EN=1;

break;

case MERES:   serou_press();

break;

case CMER_ON: CMER=1;

break;

case CMER_OFF:CMER=0;

break;

case HOMERES: temp_rd();

serou_hom();

break;

default:   sout_byte(ERROR);

}

}

if (CMER)

{

serou_press();

}

}

}